Embedded Numerical Control System Control Card with USB Terminal

Manual

# Contents

Chapter I

- Installation

  - Installation of software

  - Installation of hardware

- Description of buttons function

- Description of operating

- Diagram of menu list

- Description of software downloading

Chapter II

- Description of basic operations

  - How to number the keyboard

  - How to press buttons

  - How to reply clues

  - How to input single number

  - How to input 3 axes parameter

  - How to operate Menu

  - Long-timed storage

- Description of system Mode

  - Manual processing state

  - Menu state

  - Auto processing state

- Description of coordinate system operating

  - Machine tool coordinate system

  - Work coordinate system

  - Coordinate system operating method

- Description of manual processing operating

  - Manual speed mode

  - Manual mesh

  - Manual moving mode

  - Description of manual processing operating

- Description of auto processing operating

  - Auto processing

  - Broken point saving

  - Matrix processing

  - Cautions

- Description of parameter setting

- Operating method of cut adjusting device

- Multi files system

- One button recovery function

X)Appendix (circuit diagram of control card)

Chapter I

## Installation

Control Card is totally separated from computer and controls caving machine independently. It receives target file through USB interface on computer. User needs to install USB driver which offers user two files: “*.inf” and “*.sys”. files’ transmission and downloading are executed by software: ZHBUSBHOST.exe. This card communicates with carving machine through 37 pins terminal. This system offers driving files and downloading files. At the same time, users connect the 37 pin terminal to communicational terminals on different carving machines.

This system is easy to install and operate, it is more stale than ISA or PCI socket control card which needs computer. It reduces troubles (such as: computer system crashes down, there is no such mainboard with socket or it does not comply with the mainboard) and makes convenient for the users.

  - Software setup

It is convenient to set up the software. To copy all files on CD-ROM root directory to the directory on your computer. For an example, copy them to D:\DSP. Then to set up USB driving software. It is necessary to prepare a USB terminal for control card. Connect the control card to the computer and then open the switch on the control card( up is “ON”, down is “OFF”). Computer searches USB driving software automatically. When it appears clues asking to choose driving file, you should choose D:\ DSP\USB\D12TEST.inf in Windows’98 or D:\DSP\USBCAM.inf in Windows’2000.If ZHBUSBHOST.exe file in DSP directory can be executed successfully; it means that the software has been downloaded correctly.

Detailed installation steps follow:

When the computer automatically finds USB driver, clue appears as below(In Window98)

There is no wanted driver in the second edition of Window98. Click “Confirm” and it displays as follow:

Input driver file directory in Dialogue window or click browser to search D12TEST.inf in Windows’98 driver files.

Click “Confirm” and it displays as below:

Re-click “Confirm”. the system goes into setup state and updates system information. It adds a line to “Universal Host Control” which is in Device Management; it is illustrated as red line circle in following figure that indicates software has been set up successfully.

The installation steps are similar to Windows’2000/me/xp/NT(for an instance in Window’s 2000):

After connect the control card to the USB interface, the following windows appears:

Then the following information appears:

Click “Confirm” according to clue, then it appears following window:

Input driver file directory in Dialogue Window or click browser to seek Win2000 drive file USBCAM.inf( which is in Win2000-xp-driver folder). Click “Confirm” then reclick, the system begins setting up.

  - Hardware Setup

The control card adopts50 pins terminal to communicate with carving machine. The detailed definition of the 50 pins are as follows:(circuit diagram is in appendix)

Connection between Control Card and carving machine

The user needs a 37 pins connecting line through which the control card communicates with carving machine. Generally speaking, carving machine only uses 12 pins within the 37 pins. So it             prepares I/O terminal for user to expand other functions; such as, auto sword adjusting, hard limit position and master axis startup/stop function. The 37 pins connecting line are as follows:

| pin | signal | Function specification |
| --- | --- | --- |
| 1 | VCC 5V | +5v |
| 2 | X PULSE+ | X axis pulse+ |
| 3 | X DIR+ | Direction of X axis+ |
| 4 | YPULSE+ | Y axis pulse+ |
| 5 | Y DIR+ | Direction of Y axis+ |
| 6 | Z PULSE+ | Z axis pulse+ |
| 7 | Z DIR+ | Direction of Z axis+ |
| 8 | C PULSE+ | C axis pulse+ |
| 9 | C DIR+ | Direction of C axis+ |
| 10 | CADI 11+ | Figure inputting terminal |
| 11 | CADI 10+ | Figure inputting terminal |
| 12 | CADI 9+ | Figure inputting terminal |
| 13 | CADI 8+ | Figure inputting terminal |
| 14 | VCC 5V | +5v |
| 15 | CADI 7+ | Figure inputting terminal; Cut adjusting device signal input+ |
| 16 | CADI 6+ | Figure inputting terminal |
| 17 | CADI 5 | Figure inputting terminal |
| 18 | CADI 3 | Figure inputting terminal |
| 19 | CADI 1 | Figure inputting terminal; origin of Y axis sensor signal |
| 20 | AVDD I | IO positive shared terminal |
| 21 | AVDD O | IO positive shared terminal |
| 22 | CADO 1 | Figure outputting terminal |
| 23 | CADO 3 | Figure outputting terminal |
| 24 | CADO 5 | Figure outputting terminal |
| 25 | CADO 7 | Figure outputting terminal |
| 26 | GND | GND |
| 27 | X PULSE- | X axis pulse- |
| 28 | X DIR- | Direction of X axis- |
| 29 | Y PULSE- | Y axis pulse- |
| 30 | Y DIR- | Direction of Y axis- |
| 31 | Z PULSE- | Z axis pulse- |
| 32 | Z DIR- | Direction of Z axis- |
| 33 | C PULSE- | C axis pulse- |
| 34 | C DIR- | Direction of C axis- |
| 35 | CADI 11- | Figure inputting terminal |
| 36 | CADI 10- | Figure inputting terminal |
| 37 | CADI 9- | Figure inputting terminal |
| 38 | CADI 8- | Figure inputting terminal |
| 39 | GND | GND |
| 40 | CADI 7- | Figure inputting terminal; Cut adjusting device signal input- |
| 41 | CADI 6- | Figure inputting terminal |
| 42 | CADI 4 | Figure inputting terminal |
| 43 | CADI 2 | Figure inputting terminal; origin of Z axis sensor signal |
| 44 | CADI 0 | Figure inputting terminal; origin of X axis sensor signal |
| 45 | CADGND | IO negative shared terminal |
| 46 | CADGND | IO negative shared terminal |
| 47 | CADO 0 | Figure outputting terminal |
| 48 | CADO 2 | Figure outputting terminal |
| 49 | CADO 4 | Figure outputting terminal |
| 50 | CADO 6 | Figure outputting terminal |

After having got know the detailed definition of the 50 pins, we should get know how to connect the control card and machine;

50pins cable

Control card

Machine

From above structure, we can tell that the 50 pins cable works as a bridge between the control card and machine. The relationship between control card and 50 pins cable is unchangable; while the relationship between 50 pins cable and machine is changeable, this is why carving machine is always lower than control card. Common carving machine just apply 12 pins among the 50 pins that is only 12 pins are useable. And this control card has I\O interface which makes it to expand other functions( such as: auto adjust cut ). The 50 pins cable connection method is illustrated as below:

3.Test CNC Engraver and control card

After having been connected with CNC engraver correctly, the control card should be opened(before doing this, pull out USB connecting line and disconnect it from computer) and the screen displays:

Goto Home?

pressI①to go upward

pressII①to go downward

pressI①to go upward

pressII①to go downward

PressIV④ to skip

pressIV①to confirm

Press I① button to confirm (Buttons’ detailed function specification are in Description of Buttons’ Function section). And then all axes move to origin of machine tool. If it doesn’t work well, press III④,the system clues to  password, input 2003 and then press III④ button to confirm. Entering function setting menu {                     }and find SYSTEM SETUP, press III④ to confirm{                     }to get Home Direction, press III④ to confirm{                     },then pressing III① makes to go to machine origin.

Then check their directions, if it moves to wrong direction, press III④ to enter function setting menu, press IV① to confirm, find SYSTEM SETUP with up or down buttons, press IV① to confirm, then find Motor Direction, set each axis direction(setting method is similar to clear direction setting). If they move to right direction, it indicated that the connection between control card and engraving machine is correct.

pressI①to go upward

pressII①to go downward

If it is necessary to adjust leading screw interspace for testing the machine, press III④,the system clues to  Password, input 2003 and then press IV① button to confirm. Entering function setting menu {                     }and find Screw Interspace, press IV① to confirm. User must be careful to set this parameter, otherwise it may do harm to the engraving machine.

When the control card connects with computer, USB connecting line supplies power to the control card; when it connects with engraving machine, and engraving machine system offers power to it. So it is necessary to disconnect one end in order to connect the other.

User may connect it to computer to examine.

Head axis startup/stop control signal terminal has not been definited yet which needs to process according to user’s demands and control mode.

In order to guarantee normal operation, carving machine’s highest speed and acceleration should set according to different carving machine.

### Description of buttons function

| I① | I② | I③ | I④ |
| --- | --- | --- | --- |
| II① | II② | II③ | II④ |
| III① | III② | III③ | III④ |
| IV① | IV② | IV③ | IV④ |

Figures above show position of buttons and their functions are illustrated as below:

Z+ ▲1 X+ 2∧ Y+ 3 XY→0 4 ►►▌/►▌ 0 Y 6∨ Z 7 9 X ▼5 Z→O 8 0 ON/OFF . Menu _ --/─ Confirm ►/▏▏ Delete ■ Cancel

Chart of buttons function is below.

| Name | Function |
| --- | --- |
| I① | Positive movement of Z axis, Menu upward selection, figure 1 inputting |
| I② | Positive movement of Y axis, accelerate cut speed, figure 2 inputting |
| I③ | Positive movement of Z axis, figure 3 inputting |
| I④ | Origin of X axis and Y axis setting, figure 4 inputting |
| II① | Negative movement of X axis; Menu downward selection figure 5 inputting |
| II② | Negative movement of Y axis; slowdown cut speed; figure 6 inputting |
| II③ | Negative movement of Z axis, figure 7 inputting |
| II④ | Z axis origin setting ; figure 8 inputting |
| III① | Axes clear to machine tool origin, figure 9 inputting |
| III② | Manual moving mode, high speed and low speed selection, figure 0 inputting |
| III③ | Head axis startup/stop, decimal point inputting |
| III④ | Menu setting entering, negative symbol inputting |
| IV① | All axes clear: motions confirm/inputting/operating |
| IV② | Manual move, continue, step and space modes selection |
| IV③ | Cut process running/pause/inputted words delete |
| IV④ | Cut process stop/selections, inputting and operating cancel |

### Description of operating

Users can program target files by programmable software, such as, TYPE3,ARTCAM ,MASTERCAM,UG.,Pro/E,CAXAand get G code files whose extended file name must be “*.CNC”. And the file will be downloaded to control card through ZBHUSBHOST.exe file which offered by the system.

Before execute ZBHUSBHOST.exe file, connect computer and control card through USB line and disconnect carving machine and control card (pulling out 50 pins plug), then push switch to “ON” position (upward) and open control card. Execute ZHUSHOST.exe file (double click file symbol). The process of downloading G code file to control card is simple. Open ZBHUSBHOST.exe file and then target G code file, choose “Setting—change process mode” and “Common mode” at the left down corner on the screen changes into “High speed mode”. Then select “Setting---Process parameter configuration” to set process speed, empty running speed and acceleration. It is unnecessary to high manual section. Finally choose “Operating---download process data” till “download Finished” dialogue window appears which means that G code file has been download completely (Details are in software download section).

After having downloaded target file, quit ZBHUSBHOST.exe file and close control card (push switch to “OFF” position) then pull out USB connecting line. Connect control card to carving machine and start the carving machine, then control card screen clues:

Clear?

Press IV① to confirm and all axes go to machine tool origin and LCD screen displays:

1X   0 Manual

1Y   0 Axis stop

1Z   0 High speed

Continuous

After clear process and having target material fixed well, then to definite origin position of target material. For example, we are going to set its center to work origin, the operating steps are as follow: 1) move X and Y axes to the center of target material, press I④ to clear the two axes; 2)move Z axis to the center of target material and press II④ to clear it. Cutting depth and cut safe high are determined by design software. LCD screen displays:

1X   0 Manual

1Y   0 Axis stop

1Z   0 High speed

Continuous

Press III② to switch high speed and low speed and the LCD displays:

1X   0 Manual

1Y   0 Axis stop

1Z   0 Low speed

Continuous

After having located work origin, pressing IV③ and LCD displays:

1X   0 Manual

1Y   0 Axis stop

1Z   0 Low speed

Speed1.000

Before displaying “Speed 1.000” at the left corner, the system will display target file name and lasts for 1 second. The speed can not accelerate if it displays “Speed 1.000” at the left corner. If it is necessary to accelerate the speed, you must set the speed before you download file. If it needs to change process speed, press I② and II② to accelerate and reduce.

Press IV③to pause cutting and re-press IV③ to continue. When it is in Pause state, there is only Z axis can move. Z axis moving leads to change cutting depth, so it is easy to reset the cutting depth when the cutting depth is not suitable.

Press IV④to stop cutting motion and LCD screen displays:

1X   0 Manual

1Y   0 Axis stop

1Z   0 High speed

Save Broken Point?

To press I① (Figure1)  saves broken point at 1 area( Broken points can be saved in 1,2,3 areas).

1X   0 Manual

1Y   0 Axis stop

1Z   0 High speed

Save Broken Point? 1

Pressing IV①confirms and LCD screen displays:

1X   0 Manual

1Y   0 Axis stop

1Z   0 Low speed

Clear?

Pressing IV① confirms and LCD screen displays:

1X   0 Manual

1Y   0 Axis stop

1Z   0 Low speed

Continuous

“X” on the screen means that the space between set Z axis working origin and machine tool origin.

Press IV③+I①starts cutting from broken point1 and IV③+I② starts cutting from broken point 2. .

System maintenance

Parameter setting

Array process

Coordinate setting

Parameter Saving

Main interface

Diagram of Menu List

The following is the menu

How to get into the main Menu, we are going to introduce as follows:

The control card displays: Clear? When it get power supply, press III① confirm and gets into Main interface, otherwise press III④deny. It displays:

1X  0.000 manual

1Y  0.000 axis stops

1Z  0.000 low speed

continuous

When it is above main interface, you can get into function menu setting, press III④ and the system clues to “Password”, input figure “2003” and the system displays“****”，right password leads to function setting menu and wrong password leads to go back main interface.

After getting know the method of getting into function setting menu, we are going to learn detailed submenu information.

Coordinate system setting

Origin coordinate

Set it to origin

Origin coordinate Unit:mm

0.00

0.00

0.00

（coordinate system setting structure diagram as above）

Array process

Array row

Array column

Array row space

Array column space

Array rows

2

Array columns

2

Array row space

Unit:mm

10

Array column space

Unit:mm

10

Parameter setting

Current speed ratio

Clear speed

Auto process setting

Software limit position

Approaching  speed ratio

Cut adjusting device position

Unit:mm/minute

1000.00

1000.00

1000.00

Current speed ratio

1.0000

Main axis switcher

Press confirm open

Press cancel close

Current ratio

1.000

Unit:mm

0.000

0.000

0.000

（array process structure diagram is as above）

Manual moving setting

Manual switch

(it is parameter setting structure diagram as above)

Auto

Process

setting

Auto process speed

Auto process acceleration

Empty running speed

Unit:mm/minute

2000.00

Unit:mm/s2

500.00

500.00

500.00

Unit:mm/minute

2500.00

(this is auto process setting diagram)

Manual moving setting

speed of mode speed

Low speed mode acceleration

High speed mode

acceleration

High speed mode mesh

Unit:mm/m

500.00

500.00

500.00

Unit:mm/s2

500.00

500.00

500.00

Unit:mm

0.100

Unit:mm/s2

500.00

500.00

500.00

Unit:mm

0.500

Low speed mode mesh

High speed mode speed

Unit:mm/m

2000.00

2000.00

2000.00

Softwar limit position

X axis

Y  axis

Z  axis

X axis SLP

-1000.00

1000.00

Y axis SLP

-1000.00

1000.00

Z axis SLP

-1000.00

1000.00

(this is manual moving setting diagram)

(this is software limit position diagram)

The data above is system default, users can set their values according to different carving machine.

System maintenace

Pulse equivalent

Screw space

Electronic machinery direction

Clear direction

Format

Unit:

pulse/mm

400.000

400.000

400.000

Unit:mm

0.000

0.000

0.000

Press confirm change direction X axis EMD+

Y axis electronic machinery direction—

Press confirm change direction

X axis clear direction—

Y axis clear direction+

Z轴回零方向+

（this is system maintenance diagram）

In system maintenance, pulse equivalent should be set according to electronic machinery, screw space’s value gets from scale, and “Electronic machinery direction” and “clearing direction” should be set by users according to different carving machine.

### Description of software downloading

It is through ZHBUSBHOST.exe file to download G code file to control card.. To open this file needs to connect control card through USB connecting line and open the switch to offer power to control card. To execute ZHBUSBHOST.exe (double click left button of mouse) runs this software as below:

Click “File→Open” and dialogue window appears:

Input target file’s name in “File name”, such as “gt03”, the extended name of the G code file must be CNC. Click “Open” and diagram of target file appears:

The next operating is to click “Setting    Change process mode”. Process modes include Common mode and High speed mode. The speed of Common mode will be very slow, especially when it does curvilinear motion. To advertisement carving machine, it is always to adopt High speed mode to meets carving decision demands.

`

“Common mode” changes into “High speed mode” at left down corner on the screen after clicking “change process mode”.

Select “Setting    Configuration process parameter” and dialogue widow appears. Input parameter into dialogue window.

Auto process speed:---it refers to running speed when carving machine starts cutting

Auto empty running speed---it refers to empty cutting speed when carving machine rise cuts.

Auto process acceleration---its value is higher and the cutting speed is faster.

Manual high speed item does not need to set which has been set in control system. Click “OK” to close dialogue window after setting. Select “Operation    Download process data” to download target file to control card. And downloading processing percentage will show. Following dialogue window appears finally which means target file has been downloaded completely.

Cautions:

Do not click “Operation      Format” which leads to format programs in control system.

“Download menu” downloads operating interface to control card. Its name is “Menu.txt” which is adjustable.

“Update” is for updating main control system, it is unnecessary for user to execute this function.

Chapter II

#### Description of basic operations

  - How to give numbers to the keyboard

They are four lines of numbers in keyboard with 4 buttons for each line. Each button has two or above functions. In different states the definition for each button is quit different (Details are in Description of buttons). In order to make it easy to remember, follows are the numbering method:   we describe buttons in the first line as I* and II* to buttons in the second line. The same to the third and fourth lines. Diagram of buttons is illustrated as below:

| I① | I② | I③ | I④ |
| --- | --- | --- | --- |
| II① | II② | II③ | II④ |
| III① | III② | III③ | III④ |
| IV① | IV② | IV③ | IV④ |

There are three ways to press buttons: it is usually to press wanted button and release at once; the second way is long-timed pressing which means to press the wanted button for 0.5 second and then release; the third way is to press combined buttons, such as pressing combined buttons II②+ IV③, its operating method steps are: firstly press and keep pressing II②then press IV③, finally release the two buttons at same time. There is no sequence to release buttons.

3.How to reply clues

Frequently there are clues when the system is running. For example, when the system starts up, it will clues: “Clear?” at the bottom of the screen. And it clues “Clear?” when it has finished process. If there are no special demands, pressing IV① confirms the clue and pressing IV④ denies. Such as: it clues “Clear?” Pressing IV① is to clear and pressing IV④is not to clear.

4.How to input single number

In following descriptions of button functions, it always needs users to input figures (e.g. when we change Manual state to space moving mode or input parameters in Menu state). Its operating methods are as follows: you can press corresponding button to input any number in process (I① to III② represents figure 1 to 0), pressing III③ inputs decimal point and  pressing III④ inputs negative sign. Caution: At some cases, it only allows to input positive number and negative sign is not allowed to input. At other cases, it only allows to input positive integer, then decimal point and negative sign are not allowed to input. If there is wrong inputting, pressing IV③ delete former inputted symbol. After inputting all figures, pressing IV① finishes inputting process or pressing IV④ cancel inputting.

For an example: it needs to input “-3129.87”, the inputting sequences are: pressing III④, I③, I①, IV①, IV③, II④, II③.

5.How to input 3 axes parameters

It frequently needs to input3 axes parameters when you set system parameter in Menu (such as Clear speed and pulse equivalent). It needs to input 3 continuous figures, the first inputted figure refers to corresponding parameter of X axis, the second represents Y axis and the third represents Z axis. The system shows current value of parameter before user input. Pressing IV① or IV④ keeps this value. If you input a new value according to single number inputting methods, the parameter changes into new value.

There is an example for inputting clear speed. Current clear speed is 1500mm/minute (X axis), 1400mm/m (Y axis) and 1300mm/m (Z axis). It needs to change clear speed to 1500mm/m (X axis),1200mm/m (Y axis) and 800.5mm/m (Z axis). Select “Clear speed” in Menu and it displays as below on the screen:

Unit: mm/minute

1500.00

“1500.00” displaying on the screen is the current clear speed of X axis, directly pressing IV① keeps its clear speed. Then the screen displays as below:

Unit: mm/minute

1500.00

1400.00

“1400.00” displaying on the screen is current clear speed of Y axis, input “1200”and the screen displays as below:

Unit: mm/minute

1500.00

1200.00

1300.00

“1200.00” on the screen is new clear speed of Y axis and “1300.00” is current clear speed of Z axis. Inputting“800.5” changes clear speed of Z axis. Then it finishes new clear speed setting.

6. How to operate Menu

After opening Menu, the screen displays below:

Parameter saving

Coordinate system setting

Array process

Parameter setting

On the screen each Menu item displays from the top to the bottom. The item with black background is the current chosen item. Pressing I① moves cursor to previous item and pressing II① moves cursor to next item. If the chosen item is the first item in the menu, pressing I① moves cursor to the last item, the same to last item, if it is the current chosen item Pressing II① moves cursor to the first item. Because of screen space limit, it can not display all items. When Pressing I① and II① choose menu item, the system rolls the screen automatically and it makes that the chosen item is on the screen.

After selecting wanted item, pressing IV① executes its function. If it has sub-menu, pressing IV① enters the sub-menu

When it displays Menu, pressing IV④ goes back previous item. For an example, user chooses “Coordinate system setting” and press IV① to go into its sub-menu, at this time, pressing IV④ goes back “Coordinate system setting”. Attention: long-timed pressing IV④ quits Menu displaying state and goes into Manual moving state (Detailed information is in “Description of system state” section).

7.Long-timed storage

It is frequently to meet this term in this manual. So-called long-timed storage means that every change of the value will be stored even the control card closes or power falls. If it is formatted, the value goes back fault.

#### Description of system state

The system has several different states. It displays different content on the screen in different states and buttons have different function. So it is necessary to understand which state the system is in.

  - Manual process state

When the system starts up, it displays as follow after executing clear operating:

1X 0.000 Manual

1Y 0.000 Axis stop

1Z 0.000 Low speed

Continuous

It is fundamental system state, within which users can execute many basic operations, such as: manual control moving, array process, clear, coordinate system changing, origin setting and broken point recovery and so on.

  - Menu state

When it is in manual process state, pressing III④ and inputting passwords (the operating method is the same to single figure inputting method). If it is wrong password, it returns Manual process state; if it is correct password, the screen displays:

Parameter saving

Coordinate system setting

Array process

Parameter setting

In Menu state, it can execute many menu operations and all kinds of menu demands, such as: highest acceleration setting and manual speed.

  - Auto process state

When ii is in Manual state, pressing IV③ runs into Auto process state, and the screen displays as below:

1X 0.000 Manual

1Y 0.000 Axis stop

1Z 0.000 Low speed

DBXP?

In this state, it can execute many menu operations, such as: pause, stop, save broken point and adjust speed ratio and so on.

#### Description of coordinate system operating

No matter what state the system is in, user must confirm coordinate system before start processing.

  - Machine tool coordinate system

Machine tool coordinate system is the primary one among all coordinate systems. Each machine tool just has one coordinate system. Its origin is the switch position of machine tool zero point, its X direction is the direction of X axis screw; its Y direction is the direction of Y axis screw; its Z direction is the direction of Z axis screw. It is absolute coordinate system and other coordinate systems are relative coordinate system to this coordinate system.

At most cases in order to accurate machine tool coordinate system before operating, it is necessary to execute clear operating when the system starts up (press IV① when it clues “Clear?”). After clearing, the system moves according to set direction and speed till it reaches at zero point of machine tool. Then the system stops and set the zero point to machine tool origin automatically.

    - The type of zero point switch

In order to accurate clear operating, it should always output a different sing to normal position when zero point switch moves out zero point position.

    - Install position of zero point switch

The zero point switch must be installed at a corner of the machine tool. Then it can move back by certain direction to machine tool zero point no matter what position the machine tool is.

    - Clear direction

To set clear direction according to zero point switch position promises clear moves to right direction not to opposite direction. The direction relates with zero point position and electronic machinery swear direction (details are in Description of Menu function).

    - Clear speed

In order to prove efficiency, clear speed should be set as fast as possible (eg. 3000mm/m). Clear speed for each axis is separated from each other’s because of different axis carriage or different electronic machineries. Attention: Because of adopting unique algorithm, clear decision has nothing to with clear speed. (Details are in Menu function section)

    - Clear acceleration

To set clear acceleration should base on each axis clear speed, electronic machinery and axis carriage. If Clear speed is faster then clear acceleration becomes slower; if electronic machinery carriage is bigger, clear acceleration is slower. (Details are in Menu function section)

Machine tool coordinate system is unchangeable, so users can operate machine tool by times. Once it records the value of machine tool coordinate system, it is easy to go back that point at any time.

  - Work coordinate system

When we program G code processing, we adopt separated coordinate system in order to convenient to send materials and install card, this coordinate system is work coordinate system and all coordinate systems are relative coordinate system in G code file. It is always to install card and processed material, then to a proper point on the material to work coordinate system origin and begin to operate.

The relationships between work coordinate system and machine coordinate system are illustrated as below:

Machine tool origin

Machine coordinate system

Work coordinate system

Work origin

Work trace

Their directions are same which differs a slanting. In fact, the slanting is the workpiece origin value in machine tool coordinate system. So it is easy to set work coordinate system by setting the value of work origin in machine tool system. And their transformation formation is illustrated below:

Xa= Xo  +  Xr

Ya= Yo  +  Yr

Za= Zo  +  Zr

Xa, Ya and Za are values of machine coordinate system; Xo, Yo and Zo are values of workpiece coordinate system origin in machine coordinate system; Xr, Yr and Zr are values of work coordinate system.

There are always several target files which need to operate, so the system offers 9 work coordinate system for users. Users can freely set that 9 work coordinate system origins and it is convenient for users to switch current work coordinate systems.

If the machine tool offers power failure protection function, it will automatically save that 9 set work coordinate system even the power falls suddenly. They are available to operate when the control card opens next time.

Because auto process, array process, broken point process are relative to current work coordinate system, it is necessary to make sure that the current coordinate system is correct before operating mentioned functions. Before operating auto process, array process, broken point process, it is always to move to align point and set the align point to current work coordinate system origin (Detailed information is in “System operating method” section), then begins to run motion.

  - Coordinate system operating method

We number the 9 work coordinate system fro figure 1 to 9. in manual process state and auto process state, we can tell current work coordinate system by checking the first figures in the first line to the third line on the screen. Illustration is below:

1X 0.000 Manual

1Y 0.000 Axis stop

1Z 0.000 Low speed

Continuous

The figure “1” on the left shows that current work coordinate system is NO.1 work coordinate system. If it is “A” on the left, it means that machine tool system is current coordinate system. In Manual process state, to press III④ + any figure form 1 to 9 changes current work coordinate systems among the 9 coordinate systems. For an example, it needs to change current coordinate system into 7th work coordinate system, the operating method is to press III④ + II③ and the screen displays:

7X 0.000 Manual

7Y 0.000 Axis stop

7Z 0.000 Low speed

Continuous

If you want to check value of machine tool coordinate system, press III④ + III② switch between current work coordinate system and machine tool coordinate system. For an example: it is in 7th coordinate system, the screen displays while pressing III④ + III②:

AX 0.000 Manual

AY 0.000 Axis stop

AZ 0.000 Low speed

Continuous

Repressing III④ + III② switches to 7th work coordinate system and the screen displays as below:

7X 0.000 Manual

7Y 0.000 Axis stop

7Z 0.000 Low speed

Continuous

Caution: It does not change current work coordinate system but its display state when we switch display states between current coordinate system and machine tool coordinate system.

In manual process state, we can not only switch current work coordinate system but also set origin position of current coordinate system. Pressing I④ sets X, Y current position to X axis and Y axis values of current coordinate system origin. Pressing II④ set Z position to Z axis value of current coordinate system origin. Follow is an example: It is in 5th coordinate system, the current origin value is at (20.0, 30.0, 40.0) and current coordinate position is at (100.5, 120.2, 220.45). According to mentioned formation, current machine tool coordinate system position is at (120.5, 150.2, 260.45). Then the screen display as below:

5X 100.500 Manual

5Y 120.200 Axis stop

5Z 220.450 Low speed

Continuous

Pressing I④ sets current machine tool coordinate system X,Y position to X,Y value of 5th work coordinate system origin. At this time the screen displays:

5X 0.000 Manual

5Y 0.000 Axis stop

5Z 220.450 Low speed

Continuous

Pressing II④ sets current machine tool coordinate system Z position to Z value of 5th work coordinate system origin. At this time the screen displays:

5X 0.000 Manual

5Y 0.000 Axis stop

5Z 0.000 Low speed

Continuous

Then current machine tool coordinate position (120.5, 150.2, 260.45) becomes 5th work coordinate system origin.

It is the short way to set work coordinate system origin, and another way is to input work coordinate system origin directly with coordinate system inputting method (Details are in “Description of parameter setting”).

If it has power failure protection model, the system will save current work coordinate system origin setting. All default value of work coordinate system is (0, 0, 0) and default work coordinate system is 1st work coordinate system.

##### Description of Manual process

Manual process means user directly controls machine tool’s moving direction and distance. This function is used to adjust machine tool’s position, set origin and test. This function is available only in manual moving state.

In order to get know manual process, it is necessary to understand manual moving mode, manual speed mode and manual mesh. Because manual process is controlled and produced by pressing button combines manual moving mode, manual speed mode and manual mesh.

- Manual speed mode

Manual speed can be set freely, but it is complex to do that directly (Details are in Parameter setting section). It is impossible for user to set this frequently. Generally, users have two aims for manual moving: one is to adjust machine tool position that promotes efficiency; the other is to adjust cut location which promotes precision. So the system offers two manual speed modes: one is high speed mode and the other is low speed mode. Users can set the two modes separately or pressing III② switches state between the two modes in Manual moving state. Users can check the speed mode at the down right corner on the screen.

Attention: Manual moving speed is long-timed storing variable. The default value for low speed mode is 1meter/minute and 3meters/minute for high speed mode.

- Manual mesh

In real Manual moving process, it requires to move to stable displace position. For an example: it needs to move a few mm or least distance, and it is hard for traditional manual moving control methods to do this. So here we adopts the definition-mesh which guarantees manual moving to stop at mesh point. For an example, the current mesh is 0.1mm, when a positive moving stops at 37.52mm, the system will continue to move to 37.6mm point automatically. Attention: this auto movement is relative to machine tool coordinate system.

Users can set mesh value freely whose range is from 0.05mm to 1mm. Because it is complex to set mesh space (Details are in Parameter setting section) and it just needs to set long space for high speed and short space for low speed, the system offers user to set two kinds of spaces: one is the long space in high speed whose value is bigger (eg. 0.5mm or 1mm), the other is short space in low speed whose value is smaller (eg. 0.05mm or 0.1mm). when pressing III② switches speed modes, it switches mesh spaces automatically.

- Manual moving mode

In order to meet different requirements for manual moving, it offers three kinds of manual moving modes, there are Continuous moving mode, Step moving mode and Space moving mode. Users can switch moving modes by pressing IV② in manual state and check the moving mode at bottom of the screen.

  - Continuous moving mode

In this mode, users press moving direction buttons (I①, I②, I③, II①, II② and II③) and machine tool will move as ordered direction till all buttons release. Its speed is determined by current speed mode. Attention: if the pressing time on buttons is too short (less than 0.5 second) and releases the button at once, the machine tool moves to and stops at nearest mesh point automatically. When this moving mode ends, the machine tool always stops at mesh point. This mode is suitable to adjust position of machine tool.

  - Step moving mode

It always moves at low speed with two mesh per second. Its mesh space is determined by current speed mode. This mode is suitable to adjust cut position and machine tool position precisely.

  - Space moving mode

In this mode, users press moving direction buttons (I①, I②, I③, II①, II② and II③) and machine tool moves to ordered location. The machine tool moves according to current speed mode and set spaces. The moving is not influenced by mesh and stops precisely at ordered point not at mesh point.

###### IV)Description of Manual process operating

When the control card starts and the system is in low continuous moving mode, the screen displays:

1X  0.000 Manual

1Y  0.000 Axis stop

1Z  0.000 Low speed

Continuous

At this time users can switch speed modes between low speed mode and high speed mode by pressing IV② or switch speed modes among Continuous moving mode, Step moving mode and Space moving mode by pressing III②. For an example: Pressing III②, the screen changes into:

1X  0.000 Manual

1Y  0.000 Axis stop

1Z  0.000High speed

Continuous

It is in high speed manual mode at this time. Pressing III②, the screen changes into:

1X  0.000 Manual

1Y  0.000 Axis stop

1Z  0.000 High speed

Step

The system is in Step mode, re-pressing III②, the screen displays as below:

1X  0.000 Manual

1Y  0.000 Axis stop

1Z  0.000 High speed

Space

It displays Space at the bottom, it needs users to input figures (its unit is mm). For an example, after inputting 1000.25, the screen changes into:

1X  0.000 Manual

1Y  0.000 Axis stop

1Z  0.000 High speed

Space 1000.25

It means that it successes in inputting space moving mode when screen displays with figures. Pressing any direction button, the machine tool moves for a distance of 1000.25mm in ordered direction. If it is necessary to change the space value, it must to repeat mentioned operating method that is to press Down5 for three times and input a new value.

After having set manual moving mode and manual speed mode, users can run the manual moving by pressing direction buttons. Caution: If the electronic machinery does not move, it means that manual moving acceleration is too high and the system can not bore it. Users reset its acceleration in Menu and reduce the value.

In manual moving process, it displays current position coordinate on the screen. In Continuous moving mode and Step moving mode, user can tell whether it reaches the wanted position form displaying coordinate on the screen.

#### Description of auto process

Auto process means that the system moves according to ordered trace in G code file. The following steps must be strictly followed: before running any auto process, users must download legal G code process data. Firstly the system must be in manual state, users should connect the control card to computer with G code file through USB connecting line. Secondly users download G code file to control card (Details are in Communication program operation section). Only in Manual state, G code file can e download, otherwise something wrong will happens. The system can save one G code file and the target file must be less than 15MB( for 16MB storage system) or less than 31MB (for 32MB storage system). The downloaded file will last forever till it is reformatted. Caution: If users operate followed steps without downloading any G code file, it may cause unexpected movement.

The system is ready to start after having download target G code file. Generally, users adjust cuts to the starting point by all manual process means and set that point as current work coordinate system origin.

To carving machine, after setting origin, it is necessary to raise Z axis to a certain height in order to avoid to destroy target workpiece. Then the screen displays as follow:

1X  0.000 Manual

1Y  0.000 Axis stop

1Z  5.000 Low speed

Step

At this time, to press IV③ execute auto process program. For example, user downloaded SAM01 file, the screen displays:

1X  0.000 Manual

1Y  0.000 Axis stop

1Z  5.000 Low speed

SMA01

At the bottom of the screen it displays the target file’s name. Caution: the machine tool will not start as soon as the IV③ pressed down, because the head axis needs some time to start up. The system begin starting after head axis runs stable, at most cases it needs 2 seconds. If it is not your wanted file, pressing IV④ quit auto process. When it begins to execute auto process, the screen displays:

1X 12.371 Running

1Y 37.452 Axis start

1Z –1.000 Low speed

Speed 1.000

In auto process there is “Running” displaying on the screen which means that it is operating auto process. Displaying “Speed” on the screen refers to speed proportion whose value is form 0.1 to 1.0. Machine tool real running speed is the product of set speed in G code file times speed proportion. In running state, users press I②  to increase speed proportion (once pressing add 0.1 and 1.0 is its maximum value) or press II② to reduce speed proportion ( once pressing reduces 0.1 and its minimum value is 0.1). When it finished all G code file process, the system raises cuts to machine tool coordinate system origin of Z axis and then moves to current work coordinate system origin.

When it is in process, pressing Down7 pauses auto process and pressing Down8 stops auto process.

It displays as below when it is in pause state:

1X 12.371 Running

1Y 37.452 Axis start

1Z –1.000 Low speed

Speed 1.000

At this time, Pressing I③ and II③ adjust the location of Z axis; pressing I② and II② adjust speed proportion or pressing IV④ quits auto process and goes back Manual state.

When pressing IV④ quits auto process from running state or pause state, the system clues: “Save Broken point?, illustration is as below:

1X 14.971 Running

1Y 49.213 Axis start

1Z 0.000 Low speed

Save broken point?

If it is unnecessary to save, pressing IV④ neglects the clues. If it needs to save the broken point, pressing I①,I②, I③ then IV① saves it. Pressing I① saves the broken point in 1st broken point saving area, the same to I② and I③.

After saving broken point, it clues “Clear?” and the screen displays as below:

1X 14.971 Running

1Y 49.213 Axis start

1Z 0.000 Low speed

Clear?

If users want cut edge to go to the position of current work coordinate system origin, to pressing IV① does it; pressing IV④ keeps cut edge at the current position. It goes back manual state, after clearing, the screen displays as below:

1X 0.000 Manual

1Y 0.000 Axis stop

1Z 0.000 Low speed

Continuous

Then it finishes a total auto process program.

- Broken point process

In order to restart operating while the cuts stop or stop operating during long time process, the system offers strong broken point saving and processing function.

The system offers 3 broken point saving areas, they are No.1 broken point saving area, No.2 broken point saving area and No.3 broken point saving area. Each area can save a broken point forever and the saved broken point can be recovered by times.

In above section, we have introduced broken point saving method. And we are going to introduce how to restart operating auto process from saved broken point.

There are two points for operating broken point process: firstly, the system must be in manual state, secondly the current work coordinate system is coordinate with the work coordinate system of the broken point and the operating data are the same with the data of the broken point. If it needs to operate broken point in No.1 broken point saving area, press IV③+I①  starts running state and target file name displays at the bottom. The screen displays as below:

1X 0.000 Manual

1Y 0.000 Axis stop

1Z 5.000 Low speed

SAM01

The same to auto process, broken point process delays starting for 2 seconds till head axis runs stable. It starts at the position of broken point and following steps are the same with auto process.

- Array process

The system supports array process which repeatedly executes one target G code file according to ordered rows, columns, row space and row space.

Before running array process, it is necessary to set target array parameter, such as row(s), column(s) and row space and column space (Detailed information is in parameter setting section.). Pressing Down⑦ + Down② starts array process which the system is in manual state. The following operations are the same with auto process.

- Cautions:

1)The current work coordinate system should be properly set in all state. After setting origin it is necessary to raise the cut to safe height which prevents the cut from destroying the surface of target work piece.

2)When it executes broken point process, the current work coordinate system should complies with the work coordinate system of broken point and their G code files should be the same. Broken point saving function saves the position of broken point not the left G code file, so it demands the G code file to offer data. It can download other G code file or execute other operation when it is not executing broken point process. If it needs to execute broken point process, it needs to download G code file which broken point is in.

##### Description of parameter setting

All Parameter settings are in Menu; the setting methods have been introduced in previous sections. Here we are going to introduce long-timed saving parameters. Pressing III④ after entering Menu, follows are the definitions of those parameters:

Origin coordinate: In “coordinate system setting” menu, user can check current work coordinate origin or change current work coordinate system origin value.

Array row: In “Array process” part, array row refers to the number of rows which the system needs to repeat operating. That is how many times the system should move to the direction of Y axis. Its value should be any integer which is not less than 1.

Array column: In “Array process” part, array column refers to the number of columns which the system needs to repeat operating to the direction of X axis. Its value should be any integer which is not less than 1.

Array row space: which refers to the space between two rows to the direction of Y axis. Its unit is mm.

Caution: space here does not refer to their interval. Its definition is illustrated as below:

Array row space

Array column space: which refers to the space between two columns to the direction of X axis. Its unit is mm. And its definition is similar with array row space.

Current speed ratio: which refers to speed proportion, its value is any real number from 0.1 to 1.0.

Clear speed: which means the maximum moving speed of each axis when it executes clear operation. Its unit is mm per minute. In order to promote clear efficiency, the value should be enlarged as possible. Its moving speed is strict by electronic machinery and machine tool structure, it has nothing to do with clear decision.

Low speed mode: which refers to manual process running speed in manual low speed mode. Its unit is mm per minute. Its function specification is in Description of Manual process.

Low speed acceleration: it refers to manual acceleration in manual low speed mode. Its unit is mm/square second. It just needs to adjust once according to the character of electronic machinery and machine tool.

Low speed mode mesh: which refers to mesh space in manual low speed mode. Its unit is mm per square second. Its detailed information is in “Description of manual process”.

Speed in High speed mode: which refers to manual process running speed in high speed mode. Its unit is mm per minute. Detailed information is in Description of manual process.

High speed mode acceleration: which refers to manual acceleration in manual high speed mode. Its unit is mm per square minute. It needs to adjust once according to the character of electronic machinery and machine tool.

High speed mode mesh: which refers to mesh space in manual high speed mode. Its unit is mm. Detailed information is in Description of manual process.

Auto process mode: which refers to maximum speed of auto process. Its unit is mm per minute. Attention: the communication program in computer adopts this parameter too.

Auto process acceleration: it refers the maximum acceleration in auto process. Its unit is mm per square second. The communication program in computer adopts this parameter too.

Empty running speed: it indicates the maximum empty running speed in auto process state. Its unit is mm per minute.

X axis software position limit: it needs to input maximum and minimum value of X axis. Its unit is mm. The inputted coordinate position is absolute coordinate position. It indicates that the software limits machine tool moving range. This function works as a hardware limit switch. It is suitable for simple machine tool without hardware position limit.

Y axis software position limit: it needs to input maximum and minimum value for Y axis.

Z axis software position limit: it needs to input maximum and minimum value for Z axis.

Pulse equivalent: which refers to pulse equivalent of each axis. Its unit is equivalent per mm and its value is any integer. Its calculation method is: pulse equivalent is equal to equivalent per circuit of electronic machinery/ screw space between axes.

Screw space: which refers to the space between screws. Its unit is mm.

Electronic machinery direction: which adjusts the relationship between electronic machinery and the value of coordinate. For example: if value of X axis is adding, the electronic machinery revolves in clockwise. If it needs to change electronic machinery direction when the value of X axis is adding. Detailed operating steps are as follow:  to execute “Electronic machinery direction” function according to mentioned method, and the screen displays as below:

X axis electronic direction +

If it needs to change electronic machinery direction at this time, pressing IV① will achieve this and the “+”symbol in the last line change into “-”(Illustration is below). Pressing IV④ omits it. It is the way to operate axis direction.

X axis electronic direction –

Y axis electronic direction +

Clear direction: which refers to moving direction of machine tool while executing clear operation. Its value is determined by electronic machinery moving direction and clear switch fixed position. Its setting method is the same to mentioned electronic machinery direction setting method.

Formation: It may have bad trances after long time operating. It will be fine after being formatted. Generally speaking it is unnecessary for user to format it. If the communication program demands to do this, users do it as the clues ask.

Caution: formation leads to lose all current parameters and target G code file. It needs to reset the system and re-download G code file.

#### Cut adjusting device operating method

Firstly put the cut adjusting device on a stable work platform, then move X, Y axes to the position above the cut adjusting device, check X,Y axes’ moving data and input the values to cut adjusting device absolute position; then press decimal point button and Menu button, automatically X, Y, Z coordinates will move the position above the cut adjusting device, then Z axis will slowly cut to cut adjusting device, it stops till cut adjusting device approaching switch indicator flashes; then check changed data of Z axis and input it to cut adjusting device absolute position. If it needs to change cut during operating process, it just needs to press Menu button then Z axis auto adjusts, press confirm button after that, X, Y axes move back their form positions, repress confirm button to continue process.

#### Multi files system

It can store several target files in the control card. When user needs to process one of them, press IV③ and related figure button.

#### One button recovery function

This card offers one button recovery function when the card can’t work normally because of users’ faulty operating. Detailed operating step is: press (open/off) and  don’t let it go then insert USB power supply, then the system recovers.

#### Appendix

The brief diagram of control card is below: